### Overview

This project is a low-voltage simulation of a power grid. It generates two phase shifted sine waves and uses a custom built Zero Crossing Detector to measure the timing difference between them. This timing is then converted into a real-time Power Factor .

### The Core Logic (How it Works)

The project follows a four-step process to get from code to a Power Factor reading:

1. Generating the Waves (The Engine)

Instead of calculating complex math on the fly, I used a 32-step lookup table (an array of values).

- The code loops through this array every 625 microseconds.

- To create the phase shift, the second wave reads from the same array but starts at a different "index" (offset).

2. Signal Conditioning (The Hardware Bridge)

The Arduino outputs a square PWM signal. To make this look like a sine wave….

- I used RC Low Pass Filters to smooth the PWM into a clean sine wave.

- I used an LM358 Op-Amp as a comparator. It flip its output high the exact microsecond the sine wave crosses 2.5V (the zero-crossing point).

3. Capturing the Time (Hardware Interrupts)

I used Pins 2 and 3 as interrupt pins.

- Voltage Interrupt: When the first wave crosses zero, it saves the current time in microseconds (vtime).

- Current Interrupt: When the second wave crosses zero, it saves that time (itime).

- Sequential Logic: To ensure accuracy, the code is designed to only record the "Current" time after it has successfully seen a "Voltage" pulse. This prevents measuring the wrong gap between cycles.

4. The Math (Calculating PF)

Once we have both timestamps, the code calculates the difference.